Tilt fitting, standard
visible hinge side, for aluminium window systems with a sash overlap

1 tilt fitting, standard
with visible hinge side,
for aluminium window system with a Euro groove and sash overlap.

Leaf dimensions

max. leaf weight:		150 kg

max. leaf height:		2,600 mm

Leaf width, with 2 hinges:	370 - 1,300 mm
Leaf width with 3 hinges:	1,301 – 1,600 mm

- concealed gear for use with standard window handles.

- Two tilt stays.

- Can be designed with a vertical handle depending on the leaf height.

- Additional horizontal central interlock needed for leaves over 1,301 mm in width.

- Additional vertical central interlock needed for leaves over 1,301 mm in height.

Tests / standards

- Corrosion resistance according to DIN EN 1670, Class 5

- Certified in accordance with DIN EN 13126-8 and DIN EN 1191

Surfaces

- Al, E6/C-0 silver anodised

- Al, RAL 9016 powder-coated

- Al powder-coated in special finish

e.g. WSS tilt fitting, standard

Optional design

- Designed as a fitting for burglar-resistant windows according to
DIN EN 1627, RC2 or RC3.

Other elements needed

- Window handle: WSS catalogue for turn-tilt fittings at www.wss.de/kataloge

e.g. WSS Article:	[Enter text]

DANGER!
The max. leaf size is limited by the max. leaf weight of 150 kg. 
Please pay attention to our application diagrams.

Supplementary information
WSS catalogue for turn-tilt fittings at www.wss.de/kataloge.html

Accessories for tilt fitting, Standard

Safety element

Concealed horizontal status/lock monitor for

aluminium window systems with Euro groove.

Set consisting of

- Magnetic contact firmly installed in the frame.

- Circular magnet installed in the push rod of the leaf fitting.

- The signal contact is only displayed when the leaf is closed and the window handle is turned into the locked position. Display when the window handle is being turned.

e.g. WSS safety element – Article: 10.337.----.---

Supplementary information
WSS complete catalogue for turn-tilt fittings at www.wss.de/kataloge