Heavy duty tilt fitting, manual

concealed heavy duty bearing, for aluminium window systems with sash overlap

1 manual tilt fitting,
concealed heavy duty bearing,
for aluminium window system with a Euro groove and sash overlap.

Leaf dimensions

max. leaf weight:		180 kg

max. leaf width:		1,000 - 2,800 mm

max. leaf height:		1,000 - 2,800 mm
max. opening width:		400 mm

necessary rebate size:		21 mm

- Suitable for installation in the series profiles of various profile systems.

- Operated by a handle drive at the top.

- Windows in a landscape format are operated by two handle gears (one each on the left and right).

- Windows in a portrait format are operated by a push rod and corner linkage for vertical and horizontal locking pins (top), and an additional lift limiter.

- Concealed four-point heavy duty bearings and concealed striker closers.

- Additional locks should be used on the lock and hinge sides depending on the leaf format as well as the suction and pressure requirements.

- The opening width is fixed by two opening limiters. The operating forces are ≤ 150 N. Vertical gas springs with a suction and pressure effect are used vertically on the sides for this purpose.

- The operating forces depend on the height of the handle and have to be coordinated with the manufacturer / architect.

- All elements of corrosion-resistant materials.

Other elements needed

- Window handle - see also our complete catalogue for turn-tilt fittings at www.wss.de/kataloge.html

e.g. WSS heavy duty tilt fitting, manual

DANGER!
The max. leaf size is determined by the max. leaf weight of 180 kg.